EFFECT OF SILICA FUME AND FLY ASH MATERIAL TO THE RECYCLED CONCRETE MORTAR STRENGTH:

EXPERIMENTAL STUDY

Suharwanto1, Tirta Sutrisno 2, Nurcahya3

Universitas Wiralodra - Indramayu

suharwanto.ft@unwir.ac.id

ABSTRACT

Concrete Mortar is a mixture of cement, sand, and water that usually used in non-structural buildings, such as a brick wall, wall plaster, lean concrete, concrete decking, paving block and others, because the concrete mortar has a low strength, that is 2 to 20 MPa. The high-cost component in the concrete mortar is cement, which is 20 – 30 percent of the sand content, so the price of the mortar can high. In the other case, the sand has taken from the river or hill-sand that can devastate the environment. So the two-component of concrete mortar has been reduced to anticipate of two problems above. The cement can be reduced than be substituted by silica fume or fly ash that lower price with cement and the sand can be replaced by a recycled concrete mortar that similar characteristic with sand. The percentage of silica fume and fly ash in the experimental study are 0 – 20% from the cement content and the recycled concrete mortar is 100 % replaced the sand in the mortar. Base on the experimental study, the silica fume, and fly ash can increase the strength of the concrete mortar. The strength of recycled concrete mortar can be compared with sand concrete mortar. The percentage of the silica fume concrete in recycled concrete mortar is 10% and fly ash is 15% in order the same to the sand concrete mortar strength.

Keyword: silica fume, fly ash, sand concrete mortar, recycled concrete mortar, strength

ABSTRAK

Mortar beton merupakan campuran semen, pasir, dan air yang digunakan pada bangunan non-struktural, seperti dinding bata, dinding plester, lantai kerja, beton ganjal, paving block dan lain-lain, hal ini dikarena mortar memiliki kekuatan yang rendah, yaitu 2 hingga 20 MPa. Semen merupakan komponen yang mahal dan prosentasenya adalah 20 - 30 persen dari kandungan pasir, sehingga mortar masih cukup mahal. Dalam kasus lain, pasir yang digunakan diambil dari sungai atau pasir bukit yang dapat merusak lingkungan. Sehingga kedua komponen mortar tersebut dapat dikurangi agar kedua masalah tersebut dapat dikurangi. Semen dapat direduksi dengan silika fume atau abu terbang yang harganya lebih murah daripada semen dan pasir dapat diganti dengan mortar beton daur ulang yang memiliki karakteristik serupa dengan pasir. Persentase silica fume dan abu terbang dalam penelitian eksperimental ini adalah 0 - 20% dari kandungan semen dan kandungan mortar beton daur ulang adalah 100% menggantikan pasir dalam mortar. Berdasarkan studi eksperimental, silica fume dan abu terbang dapat meningkatkan kekuatan mortar beton. Dengan demikian, kekuatan mortar beton daur ulang dapat menyamai mortar beton yang dibuat dari pasir. Persentase beton silika fume dalam mortar beton daur ulang adalah 10% dan abu terbang adalah 15% agar kekuata antara mortar dengan pasir dan mortar daur ulang mempunyai yang sama.

Kata kunci : Slica fume, abu terbang, Mortar pasir, mortar daur ulang, kekuatan.

- Lecturer of Civil Engineering Faculty Dept. Wiralodra University – Indramayu

2)& 3) 	Student of Civil Engineering Faculty Dept.Wiralodra University - Indramayu

INTRODUCTION

Concrete mortar is a composite of cement, fine aggregate, and water. Fine aggregate is usually sand as normal concrete mortar. It is usually used for nonstructural buildings in the civil construction, such as a brick wall, wall plaster, lean concrete, concrete decking, paving block, and others because the mortar has low strength that is 2 to 20 MPa (SNI 03-6882-2002 [23] or ASTM C270 [6]). Base on the strength capacity, Concrete mortar can be divided into four types, there are: Type O is 2.4 MPa, N is 5,2 MPa, S is 12,5 MPa, and M is 17.5 MPa [6][23]. In the concrete and concrete mortar, the high price of components is cement, so it can be reduced and substituted by silica fume or fly ash material in order to make a low price. In the other case, the sand also has been taken from a river or sand-hill that can devastate the environment, and it is also can be replaced by another material that is the recycled concrete mortar [1][7][10-14][19][27]. Recycled concrete mortar has been taken from building demolish that has been crushed into fine aggregates and has characteristic similarly with sand [7][10][13] .

In several experimental studies that have been done several years ago of the additional silica fume and fly ash in the concrete and recycled concrete aggregates can increase the strength of the concrete [1][7][10-14][19][27] and how about concrete mortar? So, this research concern to the concrete mortar in especially in the recycle concrete mortar, because the research about recycled concrete mortar did not much be done and especially for additional silica fume and fly ash in the recycled concrete mortar did not much be done.

Base on the result of an experimental study by Antoni [1], Koçak [15], and other researchers [8][12][16][18][20][25-26] about silica fume and fly ash content in the sand concrete mortar can increase the compressive strength especially at an early age after. The research of Budiman [7], Ginting [10], Kesegić [13], Khalaf [14] that have research for recycled concrete mortar have result in decreasing strength when comprised of sand concrete mortar. Referring to the above researches, this research carries out the silica fume and fly ash material which is blended in the recycled concrete mortar to know the strength it. What about did the use of silica fume and fly ash in the recycled concrete mortar? Is it increases likes sand concrete mortar when using silica fume and fly ash?

EXPERIMENTAL PROGRAM

- Concrete Mortar Base Material Test

The experimental program of recycled concrete mortar has been started from taking cement, silica fume, fly ash, sand, water, and collecting a building demolish material, and then it is crushed by manual to get a fine aggregate.

Cement and silica fume have been taken from building material store near Wiralodra University - Indramayu. Fly ash is taken from PLTU Sumur Adem, Sukra, Indramayu, West Java. Sand is taken from Tomo, Sumedang, West Java.

Base on the laboratory test [2-6] and [21-24] of fly ash physical traits are: a) color is whitish gray; b) density is 2.15 g/cm3, and c) fine grain is 80% passing sieve no. 200.

Silica fume physical traits are: a) color is grayish; b) specific gravity is 0.65 g/cm3, and c) fine grain is more than 90% passing sieve no. 200.

The fine aggregate that is taken from building demolish is called recycled fine aggregate (RFA). The characteristic of it shall be similar to sand, such as gradation (grain level arrangement), water content, specific gravity, and water absorption, so they are would be tested as SNI [22] or ASTM C-136-06 [3].

Base on the result testing of fine aggregate characteristics, the sand and recycled fine aggregate can be seen in the following figure and table.

Figure 1. Sieve analysis of fine aggregate

Figure 1 indicates that all fine aggregate (sand and recycled fine aggregate or RFA) are in the limit range of the Zone II SNI [24] or ASTM standard [5], so those fine aggregate can use to make concrete mortar. The water content of sand is 11.71% and recycled fine aggregate is 19.71%. The water absorption of RFA is larger than sand. The specific gravity of sand is 2.53, and recycled aggregate is 1.88. The specific gravity of RFA is smaller than sand.

- Preparation of Specimens

The type specimen is a cube in which the dimension is 5 cm for all sides. This dimension base on SNI [23] or ASTM [6].

- Form work	b) Pouring	c) curing

Figure 2. Pouring and curing specimen

All of the specimens have made in various that based on silica fume and fly ash content (there are 0%, 5%, 10%, 15%, and 20%), and the water-cement ratio (w/c) (there are: 0.5 and 0.6).

A material proportion of mortar that makes the specimen can be seen in Tabel 1.

Tabel 1. Material proportion of mortar

| Material | Type N-w/c 0.5 (gr) | Type S-w/c 0.5 (gr) | Type M-w/c 0.5 (gr) | Type N-w/c 0.6 (gr) | Type S-w/c 0.6 (gr) | Type M-w/c 0.6 (gr) |
| --- | --- | --- | --- | --- | --- | --- |
| Cement | 309 | 323 | 337 | 309 | 313 | 329 |
| Water | 155 | 162 | 169 | 186 | 188 | 196 |
| Concrete Mortar |  |  |  |  |  |  |
| Sand | 1886 | 1857 | 1828 | 1886 | 1799 | 1769 |
| Recycled Concrete Mortar |  |  |  |  |  |  |
| RFA | 1402 | 1380 | 1359 | 1.402 | 1337 | 1314 |

Furthermore, all material proportions in Table 1 make several variations for silica fume and fly ash content there are a percentage of silica fume and fly ash are 0%, 5%, 10%, 15%, and 20% by weight of cement. It aims to know the optimum percentage content of silica fume and fly ash in the mortar.

- Crushing Test

The next step of the experimental study is a crushing test for all of the specimens that the crushing machine uses Universal Testing Machine (UTM). The UTM can be seen in Figure 3. The crushing test aims to get a compressive strength of the mortar that is an indicator of the type of concrete mortar as an above explanation.

The specimen set in the UTM (see Figure 4.a) and compresses with 2 kg/cm2/second [4] & [21] until crush (see Figure 4.b). The maximum load shall be recorded.

Figure 3. Universal Testing Machine (UTM)

- Setting up specimen 	b) Post testing

Figure 4. Testing specimen

TEST RESULT AND DISCUSSION

The concrete specimen has been measured density. The result of the measuring can be seen in Tabel 2.

Tabel 2. Average weigh/volume of concrete mortal

| Concrete mortar | Type N-w/c 0.5 (kg/m3) | Type S-w/c 0.5 (kg/m3) | Type M-w/c 0.5 (kg/m3) | Type N-w/c 0.6 (kg/m3) | Type S-w/c 0.6 (kg/m3) | Type M-w/c 0.6 (kg/m3) |
| --- | --- | --- | --- | --- | --- | --- |
| Sand concrete mortar | 2225 | 2258 | 2268 | 2221 | 2234 | 2233 |
| Recycled concrete mortar | 2218 | 2222 | 2228 | 2154 | 2156 | 2166 |

Base on the density measurement, the recycled concrete mortar has lower than sand concrete mortar. The RFA specific gravity of RFA is smaller than sand, so the density and water absorption are larger than sand. Those results can cause to the density. The absorption values of the materials can the porosity of the concrete mortar, so the density value of recycled concrete mortar smaller than sand concrete mortar.

The next step of the analysis is a crushing test. The concrete mortar specimen has been tested by crushing test to obtain the strength of concrete mortar. The concrete mortar strength result can be seen in Table 3 and 4.

Tabel 3. Crushing test of silica fume content in concrete mortar at 28 days age

| Concrete mortar | Type N-w/c 0.5 (kg/m2) | Type S-w/c 0.5 (kg/m2) | Type M-w/c 0.5 (kg/m2) | Type N-w/c 0.6 (kg/m2) | Type S-w/c 0.6 (kg/m2) | Type M-w/c 0.6 (kg/m2) |
| --- | --- | --- | --- | --- | --- | --- |
| 0% |  |  |  |  |  |  |
| Sand concrete mortar | 6.02 | 12.84 | 18.50 | 5.96 | 12.40 | 18.22 |
| Recycled concrete mortar | 5.76 | 11.54 | 18.12 | 5.52 | 11.26 | 17.07 |
| 5% |  |  |  |  |  |  |
| Sand concrete mortar | 6.51 | 13.00 | 18.70 | 6.10 | 12.80 | 18.34 |
| Recycled concrete mortar | 5.84 | 12.20 | 18.32 | 5.78 | 11.64 | 17.14 |
| 10% |  |  |  |  |  |  |
| Sand concrete mortar | 6.62 | 13.24 | 18.84 | 6.23 | 13.20 | 18.54 |
| Recycled concrete mortar | 5.98 | 12.54 | 18.70 | 5.96 | 12.28 | 17.26 |
| 15% SF |  |  |  |  |  |  |
| Sand concrete mortar | 6.56 | 13.12 | 18.73 | 6.15 | 12.90 | 18.41 |
| Recycled concrete mortar | 5.28 | 12.28 | 18.38 | 5.90 | 12.12 | 16.90 |
| 20% SF |  |  |  |  |  |  |
| Sand concrete mortar | 6.10 | 12.90 | 18.55 | 6.00 | 12.50 | 18.20 |
| Recycled concrete mortar | 5.24 | 12.14 | 18.24 | 5.22 | 11.76 | 16.72 |

Tabel 4. Crushing test of fly ash content in concrete mortar at 28 days age

| Concrete mortar | Type N-w/c 0.5 (kg/m2) | Type S-w/c 0.5 (kg/m2) | Type M-w/c 0.5 (kg/m2) | Type N-w/c 0.6 (kg/m2) | Type S-w/c 0.6 (kg/m2) | Type M-w/c 0.6 (kg/m3) |
| --- | --- | --- | --- | --- | --- | --- |
| 0% |  |  |  |  |  |  |
| Sand concrete mortar | 6.02 | 12.84 | 18.50 | 5.96 | 12.40 | 18.22 |
| Recycled concrete mortar | 5.76 | 11.54 | 18.12 | 5.52 | 11.26 | 17.07 |
| 5% |  |  |  |  |  |  |
| Sand concrete mortar | 6.10 | 12.91 | 18.62 | 6.02 | 12.52 | 18.24 |
| Recycled concrete mortar | 5.86 | 11.90 | 18.52 | 5.61 | 11.46 | 17.13 |
| Concrete mortar | Type N-w/c 0.5 (kg/m2) | Type S-w/c 0.5 (kg/m2) | Type M-w/c 0.5 (kg/m2) | Type N-w/c 0.6 (kg/m2) | Type S-w/c 0.6 (kg/m2) | Type M-w/c 0.6 (kg/m3) |
| 10% |  |  |  |  |  |  |
| Sand concrete mortar | 6.53 | 13.04 | 18.73 | 6.17 | 12.97 | 18.42 |
| Recycled concrete mortar | 5.93 | 12.13 | 18.70 | 5.77 | 12.02 | 17.22 |
| 15% SF |  |  |  |  |  |  |
| Sand concrete mortar | 6.56 | 13.18 | 18.82 | 6.19 | 12.96 | 18.41 |
| Recycled concrete mortar | 5.88 | 12.53 | 18.63 | 5.83 | 12.30 | 17.23 |
| 20% SF |  |  |  |  |  |  |
| Sand concrete mortar | 6.11 | 12.92 | 18.64 | 6.06 | 12.53 | 18.33 |
| Recycled concrete mortar | 5.13 | 11.75 | 17.88 | 4.42 | 11.21 | 16.87 |

All values of the crushing or compressive test in Tables 3 and 4 can be made a curve to see ascending or descending strength value of concrete mortar and accession strength value to standard value that has been write on the above chapter as ASTM [6] & SNI [21]. The curve of concrete mortar strength versus the percentage of silica fume and fly ash content in the concrete mortar can be seen in Figure 5 to 8. All of the curves illustrate for each strength value of concrete mortar and those curves also can signify the peak strength for each specimen that different percentages of silica fume and fly ash content. The peak value indicates the optimum value of silica fume and fly ash content in the mortar.

The names of several specimens as shown in the curves are:

- SCM-N-w/c 0.5: SCM is Sand Concrete Mortar; N is type of mortar that is type N; w/c 0.5 is a water-cement ratio. The type of line is a straight line with a square mark.

- SCM-S-w/c 0.5: SCM is Sand Concrete Mortar; S is a type of mortar that is type S; w/c 0.5 is a water-cement ratio. The type of line is a straight line with a triangle mark.

- SCM-M-w/c 0.5: SCM is Sand Concrete Mortar; S is a type of mortar that is type M; w/c 0.5 is a water-cement ratio. The type of line is a straight line with a round mark.

- RCM-N-w/c 0.5: RCM is Recycled Concrete Mortar; N is a type of mortar that is type N; w/c 0.5 is a water-cement ratio. The type of line is a dash-dot line with a cross mark.

- RCM-S-w/c 0.5: RCM is Recycled Concrete Mortar; S is a type of mortar that is type S; w/c 0.5 is a water-cement ratio. The type of line is a dash-dot line with a star mark.

- RCM-M-w/c 0.5: RCM is Recycled Concrete Mortar; S is a type of mortar that is type M; w/c 0.5 is a water-cement ratio. The type of line is a dash-dot line with a plus mark.

Those names are also similar for w/c 0.6.

Figure 5. Compressive strength for silica fume content in the w/c 0.5.

Figure 6. Compressive strength for silica fume content in the w/c 0.6.

Figure 7. Compressive strength for fly ash content in the w/c 0.5.

Figure 8. Compressive strength for fly ash content in the w/c 0.6.

All of the curves indicate which the strength of recycled concrete mortar is lower than the sand concrete mortar. That cause that the recycle fine aggregate water absorption is higher than sand and a specific gravity of recycled fine aggregate is a lower value than sand. Those characteristics may cause more porous in the concrete mortar and those also cause density of recycled concrete mortar lower value than sand concrete mortar.

The w/c 0.5 of sand concrete mortar strength can reach above the target strength standard, but the recycled concrete mortars strength cannot suitable to the target strength standard for all various silica fume and fly ash that an original recycled concrete mortar that is 0% silica fume and fly ash content. So, the original recycled concrete mortar has not recommended, except for the lower strength.

The w/c 0.6 of sand concrete mortar strength can meet above the target strength standard, but all of the recycled concrete mortar strength cannot meet the target strength standard. So the w/c 0.6 of recycled concrete mortar has not been recommended. So, the w/c 0.6 is not recommended for construction material.

Furthermore, those curves can be analyzed to get the optimum value of silica fume and fly ash content in the concrete mortar and strength values target to the standard. Base on the curves, the optimum strength values of concrete mortar are 10% content of silica fume and 15% content of fly ash for sand concrete mortar. The recycled concrete mortar can increase the strength to the standard when it is added by 10% silica fume and 15% fly ash, so the recycled concrete aggregate can be used for non-structural material that it is similarly characteristic with san concrete mortar.

CONCLUSION

Based on the experimental study, conclusions can be taken in the following sentences:

- The water absorption of recycled fine aggregate is higher than sand and the specific gravity is lower than sand. Those can affect the porosity that is indicated with a lower value of density of concrete mortar. The density of recycled concrete is lower than sand concrete mortar.

- Specimens have been made with two several of water-cement ratios (w/c) those are 0.5 and 0.6. Base on the experimental result. The recycled concrete mortar strength with w/c 0.6 did not reach to the strength standard. So, a recycled concrete mortal cannot be used for nonstructural building material, because it cannot reach to strength standard. So, recycled concrete mortar with w/c 0.6 is not recommended.

- The experimental result of silica fume and fly ash optimum content in the concrete mortar are 10% for silica fume and 15% for fly ash. Those percentages obtain peak strength for each variation of a mixture of concrete mortar. So, those percentage can be used for all mixture of sand and recycle concrete mortar.

REFERENCES

- Antoni, Chandra. L., Hardjito, D. “The impact of using fly ash, silica fume and calcium carbonate on the workability and compressive strength of mortar”, The 5th International Conference of Euro Asia Civil Engineering Forum (EACEF-5), Procedia Engineering 125, 773 – 779, 2015.

- ASTM C 230, “Standard specification for flow table for use in tests of hydraulic cement”. ASTM International, 2013.

- ASTM C 618 – 05, “Standard Specification for Coal Fly Ash and Raw or Calcined Natural Pozzolan for Use in Concrete” ASTM International, 2005.

- ASTM C109 / C109M-20a, “Standard Test Method for Compressive Strength of Hydraulic Cement Mortars (Using 2-in. or (50-mm) Cube Specimens)”, ASTM International, West Conshohocken, PA, 2020.

- ASTM C136-06/C136M-14. “Sieve Analysis of Fine and Coarse Aggregates”. ASTM. Philadelphia, USA, 2006.

- ASTM C270, “Standard Specification for Mortar for Unit Masonry”, ASTM. Philadelphia, USA, 2006.

- Budiman, “Limbah Beton Sebagai Material Agregat Halus Terhadap Kuat Tekan Karakteristik”, INTEK Jurnal Penelitian, Volume Vol. 6, No. 2, pp.145-149, 2019.

- Cheng-yi, H., Feldman, R.F., “Influence of Silica Fume on the Microstructural Development in Cement Mortars”, Cement and Conccrete Research, Vol. 15, pp. 285-294, 1985.

- Corinaldesi, V., Moriconi, G. “Behaviour of cementitious mortars containing different kinds of recycled aggregate”, Construction and Building Materials 23, pp. 289–294, 2009.

- Ginting, A., “Penggunaan Pecahan Tembok yang Didaur Ulang menjadi Pasir Buatan untuk Mortar Pasangan Batu Bata”, Jurnal Wahana Teknik. Vol. 9, No. 2, Agustus. 2007.

- Hansen TC. “Recycling of demolished concrete and masonry”. RILEM TC 37-DRC-Demolition and Reuse of Concrete. London: E and FN Spon; 1992.

- Jalilifar, H., Sajedi, F. Afshar, R., “Exprimental Effect of Using Silica Fume and Fly Ash on Mechanical Properties of Recycled Concretes”, International Journal of Engineering and Applied Sciences (IJEAS), Vol. 3, November 2016.

- Kesegić,I., Netinger, I., Bjegović, D. “Recycled clay brick as an aggregate for concrete: Overview”, Tehnicki Vjesnik, Vol. 15, No. 3, pp.35-40, October 2008.

- Khalaf, F.M., "Using Crushed Clay Brick as Coarse Aggregate in Concrete," Journal of Materials in Civil Engineering, vol. 18, no. 4, pp. 518-526, 2006.

- Koçak, Y. “A study on the effect of fly ash and silica fume substituted cement paste and mortars”, Scientific Research and Essays Vol. 5(9), pp. 990-998, 4 May, 2010.

- Liu, W., at all. “Effect of Silica Fume and Fly Ash on Compressive Strength and Weight Loss of High Strength Concrete Material in Sulfuric and Acetic Acid Attack. Engineering Materials, 748, 301–10, 2017.

- Neville AM. “Proporties of Concrete”. Pearson Education Limited. England, 2006.

- Priyanka, Bhat, M.M, “Study the Effect of Fly Ash, Silica Fume and Recycled Aggregate on the Compressive Strength of Concrete”, IJREAT International Journal of Research in Engineering & Advanced Technology, Vol. 3, Issue 1, Mach, 2015.

- Rao, A., Jha, K. N., Misre, S. "Use of Aggregates from Recycled Construction and Demolition Waste in Concrete," Resources, Conservation & Recycling, vol. 50, no. 1, pp. 71-81, 2007.

- Sasiekala. K., Malathy, R., “Behaviour of Mortar Containing Silica Fume and Fly Ash Used for Ferro-cement Laminates” , Jr. of Industrial Pollution Control, Vol. 28, pp 13-20, December, 2012

- SNI 03-6825-2002, “Metode Pengujian Kekuatan Tekan Mortar Semen Portland untuk Pekerjaan Sipil”, 2002

- SNI 03-2495, Spesifikasi Bahan Tambahan Untuk Beton”, 1991.

- SNI 03-6882-2002, "Spesifikasi Mortar untuk Pekerjaan pasangan”, 2002.

- SNI 1970:2008. “Tentang Cara Uji Berat Jenis dan Penyerapan Agregat Halus:, 2008.

- Wenno, R. Wallah, S.E., Pandaleke, R. “Kuat tekan Mortar dengan Menggunakan Abu terbang (Fly Ash) asal PLTU Amurang Sebagai Subtitusi Parsial Semen”, Jurnal Sipil Statik Vol.2 No.5, pp. 252-259, Juli, 2014.

- Yilmaz B, Olgun A. “Studies on Cement and Mortar Containing Low-Calcium Fly Ash, Limestone, and Dolomitic Limestone”. Cement Concrete Compos vol 30, No. 1, pp. 194-201, 2008.

- Yusra. A, Opirina, L. Irwansyah, “Pengaruh Subtitusi Agregat Buatan (Beton Daur Ulang) Terhadap Kuat Tekan Beton Normal”, Jurnal Teknik Sipil, Universitas Teuku Umar, Volume 5. No.1 Pp. 1 dari 12, April, 2019.